- Multi-instantiable and re-entrant safe driver

- Modeled after TI Device Driver Architecture for Stream Class Devices that allows for easy porting and customization

Description

- This EDMA3 Driver has been developed for the following h/w and s/w environment

Hardware:

- Target Boards: DA830 EVM, TCI6608 (Little and Big Endian) Simulator, TCI6616 (Little and Big Endian) Simulator, C6748 EVM, OMAPL138 EVM, C6472 (Little and Big Endian) EVM, TCI6486 (Little and Big Endian) EVM, TI816X EVM, TI814X EVM, TI811X EVM. C6670 EVM, C6678 EVM, tci6614 EVM, tci6614 Simulator, Vayu EVM (DRA7xx).

- Emulation Setup: TI Emulator.

- Cabling: Standard Serial Connector

Software:

- CCS: 5.4.0

- Operating System: SYS BIOS 6.35.03.47

- C6x Code generation tools: 7.4.2

- TMS470 Code Generation Tools: 5.0.4

- Arm GCC A15 Toolchain (Linaro) 4.7.3

- ARP32 Code Generation Tools: 1.0.2

- XDC Tool chain: 3.25.02.70

- TCI6608/TCI6616 Simulator: 1.0.0

Other S/W Components Used:

- None

- Capabilities

The DSP BIOS EDMA3 Driver adopts a scalable architecture that eases customization/extension

- Isolates H/W and OS Accesses, Easy to maintain & re-target to new platforms

- Can stack custom-functions along control/data-path to realize “driver filters”

- Uses the EDMA3 Resource Manager services

- Supports Multiple Instances

For easy and quick reconfiguration of driver for different SoCs, all its global configurable parameters can be passed at run time to the API EDMA3_DRV_create (), to create the SoC specific EDMA3 Driver Object. In case this configuration is not passed at run time, it can be taken from the EDMA3 configuration file edma3_<PLATFORM_NAME>_cfg.c, for the specific SoC, if it has been provided there. The configuration files can be found in “edma3_lld_<VERSION_NUMBER>\packages\ti\sdo\edma3\rm\src\configs” folder.

Similarly, the shadow region specific information can also be passed at run time to the API EDMA3_DRV_open (), to create region specific EDMA3 Driver Instance. In case this configuration is not passed at run time, it can be taken from the EDMA3 configuration file edma3_<PLATFORM_NAME>_cfg.c, for the specific SoC, if it has been provided there. The configuration files can be found in “edma3_lld_<VERSION_NUMBER>\packages\ti\sdo\edma3\rm\src\configs” folder.

The following table gives a quick overview of the supported API services. For help on interfaces, refer to the EDMA3 Driver Help File.

EDMA3 Driver APIs:

| EDMA3_DRV_getVersion () | The function is used to get the version information of the EDMA LLD |
| --- | --- |
| EDMA3_DRV_getVersionStr () | The function is used to get the version string for the EDMA LLD |
| EDMA3_DRV_create () | Create (and initialize) a given EDMA3 Driver (object) |
| EDMA3_DRV_delete () | Delete a given EDMA3 Driver (object) |
| EDMA3_DRV_open () | Open Instance of the EDMA3 driver |
| EDMA3_DRV_close () | Close Instance of the EDMA3 driver |
| EDMA3_DRV_requestChannel () | Request for a logical channel for data transfer |
| EDMA3_DRV_freeChannel () | Free the earlier requested logical channel |
| EDMA3_DRV_clearErrorBits () | Clean up the earlier used logical channel (For removable devices) |
| EDMA3_DRV_linkChannel () | Link two logical channels |
| EDMA3_DRV_unlinkChannel () | Unlink two earlier linked channels |
| EDMA3_DRV_chainChannel () | Chain two logical channels |
| EDMA3_DRV_unchainChannel () | Unchain two earlier chained channels |
| EDMA3_DRV_setOptField () | Set a particular subfield within the OPT field of a PaRAM Set |
| EDMA3_DRV_getOptField () | Get a particular subfield within the OPT field of a PaRAM Set |
| EDMA3_DRV_setSrcParams () | Set the Source specific parameters of the data transfer |
| EDMA3_DRV_setDestParams () | Set the Destination specific parameters of the data transfer |
| EDMA3_DRV_setSrcIndex () | Set the Source specific indices for the data transfer |
| EDMA3_DRV_setDestIndex () | Set the Destination specific indices for the data transfer |
| EDMA3_DRV_setTransferParams () | Set the various counts for the data transfer |
| EDMA3_DRV_enableTransfer () | Enable the transfer on a particular logical channel |
| EDMA3_DRV_disableTransfer () | Disable the transfer on a particular logical channel |
| EDMA3_DRV_bindPaRAMTcc () | Bind the resources PaRAM Set and TCC |
| EDMA3_DRV_setQdmaTrigWord () | Set the Trigger word for a QDMA channel |
| EDMA3_DRV_getPaRAM () | Get the PaRAM Set associated with a logical channel |
| EDMA3_DRV_setPaRAM () | Set the PaRAM Set associated with a logical channel |
| EDMA3_DRV_setPaRAMEntry () | Set a particular PaRAM set entry of the PaRAM set associated with the specified logical channel |
| EDMA3_DRV_getPaRAMEntry () | Get a particular PaRAM set entry of the PaRAM set associated with the specified logical channel |
| EDMA3_DRV_setPaRAMField () | Set a particular PaRAM set field of the PaRAM set associated with the specified logical channel |
| EDMA3_DRV_getPaRAMField () | Get a particular PaRAM set field of the PaRAM set associated with the specified logical channel |
| EDMA3_DRV_setEvtQPriority () | Sets EDMA TC priority. User can program the priority of the Event Queues at a system-wide level. This means that the user can set the priority of an IO initiated by either of the TCs (Transfer Controllers) relative to IO initiated by the other bus masters on the device (ARM, DSP, USB, etc) |
| EDMA3_DRV_mapChToEvtQ () | Associate Channel to Event Queue |
| EDMA3_DRV_getMapChToEvtQ () | Get the Event Queue mapped to the specified DMA channel |
| EDMA3_DRV_setCCRegister () | Set the Channel Controller (CC) Register value, by specifying the register offset and the new value. |
| EDMA3_DRV_getCCRegister () | Get the Channel Controller (CC) Register value, by specifying the register offset. |
| EDMA3_DRV_waitAndClearTcc () | Wait for a transfer completion interrupt to occur on the specific TCC. |
| EDMA3_DRV_checkAndClearTcc () | Returns the status of a previously initiated transfer. |
| EDMA3_DRV_getPaRAMPhyAddr () | Get the PaRAM Set Physical Address associated with a logical channel. |
| EDMA3_DRV_Ioctl () | This function provides IOCTL functionality for EDMA3 Driver. |
| EDMA3_DRV_getInstHandle | Return the previously opened EDMA3 Driver Instance handle |
| EDMA3_DRV_disableLogicalChannel | Disable the event driven DMA channel or QDMA channel |
| EDMA3_DRV_registerTccCb | Registers a transfer completion handler for a specific DMA/QDMA channel |
| EDMA3_DRV_unregisterTccCb | Un-register the previously registered callback function against a DMA/QDMA channel |
| EDMA3_DRV_setTcErrorInt | Enable/disable specific EDMA3 Transfer Controller Interrupts |
| EDMA3_DRV_getChannelStatus | Get the current status of the DMA/QDMA channel |
| EDMA3_DRV_mapTccLinkCh | Associates a link channel and a TCC |
| EDMA3_DRVinitXbarEventMap () | This function provides the functionality to map cross bar event to EDMA channels. |

EDMA3 Driver Performance Characteristics

| S No | Target | Code | Data (In bytes) |  | Total |
| --- | --- | --- | --- | --- | --- |
|  |  | (In bytes) | Initialized | Uninitialized | (In bytes) |
| 1 | C64X+ | 21120 | 584 | 57146 | 78850 |
| 2 | C67x | 21280 | 576 | 57144 | 79000 |

- Memory Usage Statistics for tda2xx M4 and C66 Core

|  | CODE (In bytes) | Data (In bytes) |  | Total (In bytes) |
| --- | --- | --- | --- | --- |
| Library for M4 |  | Initialized | Uninitialized |  |
| edma3_lld_drv_sample.aem4 | 1468 | 13084 | 52 | 14604 |
| edma3_lld_drv.aem4 | 9392 | 55 | 57140 | 66587 |
| edma3_lld_rm.aem4 | 11704 | 11462 | 48364 | 71530 |
| edma3_lld_rm_sample.aem4 | 1328 | 3236 | 52 | 4616 |
| Library for DSP |  |  |  |  |
| edma3_lld_drv_sample.ae66 | 2688 | 13120 | 12 | 15820 |
| edma3_lld_drv.ae66 | 21280 | 576 | 57114 | 79000 |
| edma3_lld_rm.ae66 | 27264 | 12523 | 48388 | 88175 |
| edma3_lld_rm_sample.ae66 | 1376 | 40 | 8 | 1424 |
| Library for EVE |  |  |  |  |
| edma3_lld_drv_sample.aearp32f | 2020 | 13131 | 52 | 15203 |
| edma3_lld_drv.aearp32f | 13208 | 593 | 57140 | 70941 |
| edma3_lld_rm.aearp32f | 15020 | 12363 | 48364 | 75747 |
| edma3_lld_rm_sample.aearp32f | 1920 | 3283 | 52 | 5255 |

References

[1] EDMA3 Module Hardware Specifications

[2] DSP BIOS Documentation

[3] EDMA3 Driver Documentation

Glossary

PaRAM Set	Parameter RAM Set in an EDMA3 Controller

- IMPORTANT NOTICE

Texas Instruments Incorporated and its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products and services at any time and to discontinue any product or service without notice. Customers should obtain the latest relevant information before placing orders and should verify that such information is current and complete.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI components. To minimize the risks associated with customer products and applications, customers should provide adequate design and operating safeguards.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products or services are used. Information published by TI regarding third-party products or services does not constitute a license from TI to use such products or services or a warranty or endorsement thereof. Use of such information may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI

Mailing Address:

Texas Instruments

Post Office Box 655303, Dallas, Texas 75265

Copyright © 2009, Texas Instruments Incorporated

LICENSE

This work is licensed under the Creative Commons Attribution-Share Alike 3.0 United States License. To view a copy of this license, visit http://creativecommons.org/licenses/by-sa/3.0/us/ or send a letter to Creative Commons, 171 Second Street, Suite 300, San Francisco, California, 94105, USA.